- Transferring ORIC disks to the PC -

Most of you EUPHORIC users know that the ORICLINK utility can be

used to transfer files stored on a tape to the PC. It relies on a

cable described in the ORICLINK readme files.

Based on the very same cable here is another utility I've called

DISKLINK that allows you to transfer a full ORIC floppy to your PC.

DISKLINK is actually two programs, a SEND program running on the

ORIC and a RECEIVE program running on the PC.

Two versions of the SEND program are provided, one for the MICRODISK

drive with ORIC-DOS, the other for the JASMIN drive with FT-DOS.

The only difference between the two versions is about :

- the diskette parameters (e.g. number of sectors per track)

- the way to read a sector into memory

When the transfer starts, the diskette parameters are also sent to

the RECEIVE program; therefore only one version is needed.

Feel free to adapt the SEND program for other operating systems and

let me know !

Interested ? OK, here goes :

0 - If not yet done, build the ORICDISK cable ...

1 - Enter the SEND program for the OS you want. You can skip the

assembly column since it is all commented out. Of course you

_must_ skip the C like comments. Just in case you made a typo,

do _not_ run the program before saving it to a floppy ;)

2 - Run DISKLINK on the PC. The program expects a file name on the

command line such as :

DISKLINK MYDISK.IMG

In case you do not provide a name DISKLINK will default to the

ORICDISK.IMG file name. The IMG extension is only my convention,

use anything you like.

3 - Insert the wanted floppy in the ORIC drive. DISKLINK transfers

one side at a time.

4 - RUN the ORIC program (you did SAVE it didn't you ? ;)

If all goes well you should see the parameters of the diskette

that DISKLINK is expecting and that it is indeed receiving data

from the track and sector numbers it displays. The ORIC program

doesn't display anything.

If for some reason you want to interrupt the transfer, hit any

key on the PC keyboard. The ORIC will loop forever until you

press the RESET button.

5 - If you are using EUPHORIC version 0.91+ then run the ORIC2MFM

utility to convert your disk file to an MFM disk format file.

Disk file is now ready !

Repeat steps 2 .. 5 for more ORIC disk sides.

Legal stuff

DISKLINK is free. You can do everything you want with it,

including modifying it and making it available to anyone by

any means.

=== ORIC side of DISKLINK for MICRODISK + ORIC-DOS ================

/* SETUP */

1 DR=0               ' DRIVE

2 T0=0               ' TRACK  MIN

3 T1=39              ' TRACK  MAX

4 S0=1               ' SECTOR MIN

5 S1=16              ' SECTOR MAX

/* Get one sector in RAM buffer */

100 DATA#4C,#08,#10  '   JMP A

110 DATA#00          '   DRIVE

120 DATA#00          '   TRACK

130 DATA#00          '   SECTOR

140 DATA#00,#00      '   ADDRESS

150 DATA#08          'A  PHP

160 DATA#A9,#80      '   LDA #$80

170 DATA#20,#E6,#04  '   JSR $04E6

180 DATA#28          '   PLP

190 DATA#A2,#04      '   LDX #$04

200 DATA#BD,#03,#10  'B  LDA $1003,X

210 DATA#9D,#00,#C0  '   STA $C000,X

220 DATA#CA          '   DEX

230 DATA#D0,#F7      '   BNE B

240 DATA#20,#24,#D4  '   JSR $D424

250 DATA#08          '   PHP

260 DATA#A9,#82      '   LDA #$82

270 DATA#20,#E6,#04  '   JSR $04E6

280 DATA#28          '   PLP

290 DATA#60          '   RTS

/* No longer used; don't skip DATAs ;) */

400 DATA#4C,#48,#10  '   JMP F

/* Send one nibble to the PC through the Oriclink cable */

410 DATA#8D,#01,#03  'C  STA $0301

420 DATA#AD,#00,#03  '   LDA $0300

430 DATA#49,#10      '   EOR #$10

440 DATA#8D,#00,#03  '   STA $0300

450 DATA#AD,#0D,#03  'D  LDA $030D

460 DATA#29,#02      '   AND #$02

470 DATA#F0,#F9      '   BEQ D

480 DATA#60          '   RTS

/* Send a whole byte to the PC */

490 DATA#48          'E  PHA

500 DATA#20,#28,#10  '   JSR C

510 DATA#68          '   PLA

520 DATA#4A          '   LSR A

530 DATA#4A          '   LSR A

540 DATA#4A          '   LSR A

550 DATA#4A          '   LSR A

560 DATA#20,#28,#10  '   JSR C

570 DATA#60          '   RTS

/* Send one sector ie 256 bytes to the PC */

580 DATA#78          'F  SEI

590 DATA#A2,#00      '   LDX #$00

600 DATA#BD,#00,#20  'G  LDA $2000,X

610 DATA#20,#3B,#10  '   JSR E

620 DATA#E8          '   INX

630 DATA#D0,#F7      '   BNE G

640 DATA#58          '   CLI

650 DATA#60          '   RTS

/* Put machine code in RAM */

700 FORI=#1000TO#1055

710 READA:POKEI,A

720 NEXT

/* Send diskette parameters to PC */

800 DOKE#2000,#100   ' SECTOR SIZE

810 DOKE#2002,S0     ' SECTOR MIN

820 DOKE#2004,S1     ' SECTOR MAX

830 DOKE#2006,T0     ' TRACK  MIN

840 DOKE#2008,T1     ' TRACK  MAX

850 CALL#1048        ' TRANSFER

/* Transfer whole diskette */

900 FOR T = T0 TO T1

910 FOR S = S0 TO S1

920 GOSUB 1000       ' READ SECTOR

930 CALL#1048        ' TRANSFER

940 NEXTS

950 NEXTT

960 END

/* Read sector for MICRODISC + ORIC-DOS */

1000 POKE#1003,DR     ' DRIVE

1010 POKE#1004,T      ' TRACK

1020 POKE#1005,S      ' SECTOR

1030 DOKE#1006,#2000  ' BUFFER ADDRESS

1040 CALL#1000        ' READ SECTOR

1050 RETURN

=== ORIC side of DISKLINK for JASMIN + FT-DOS =====================

/* SETUP */

1 DR=1               ' DRIVE

2 T0=0               ' TRACK  MIN

3 T1=39              ' TRACK  MAX

4 S0=1               ' SECTOR MIN

5 S1=17              ' SECTOR MAX

/* Send one nibble to the PC through the Oriclink cable */

410 DATA#8D,#01,#03  'C  STA $0301

420 DATA#AD,#00,#03  '   LDA $0300

430 DATA#49,#10      '   EOR #$10

440 DATA#8D,#00,#03  '   STA $0300

450 DATA#AD,#0D,#03  'D  LDA $030D

460 DATA#29,#02      '   AND #$02

470 DATA#F0,#F9      '   BEQ D

480 DATA#60          '   RTS

/* Send a whole byte to the PC */

490 DATA#48          'E  PHA

500 DATA#20,#28,#10  '   JSR C

510 DATA#68          '   PLA

520 DATA#4A          '   LSR A

530 DATA#4A          '   LSR A

540 DATA#4A          '   LSR A

550 DATA#4A          '   LSR A

560 DATA#20,#28,#10  '   JSR C

570 DATA#60          '   RTS

/* Send one sector ie 256 bytes to the PC */

580 DATA#78          'F  SEI

590 DATA#A2,#00      '   LDX #$00

600 DATA#BD,#00,#20  'G  LDA $2000,X

610 DATA#20,#3B,#10  '   JSR E

620 DATA#E8          '   INX

630 DATA#D0,#F7      '   BNE G

640 DATA#58          '   CLI

650 DATA#60          '   RTS

/* Put machine code in RAM */

700 FORI=#1028TO#1055

710 READA:POKEI,A

720 NEXT

/* Send diskette parameters to PC */

800 DOKE#2000,#100   ' SECTOR SIZE

810 DOKE#2002,S0     ' SECTOR MIN

820 DOKE#2004,S1     ' SECTOR MAX

830 DOKE#2006,T0     ' TRACK  MIN

840 DOKE#2008,T1     ' TRACK  MAX

850 CALL#1048        ' TRANSFER

/* Transfer whole diskette */

900 FOR T = T0 TO T1

910 FOR S = S0 TO S1

920 GOSUB 1000       ' READ SECTOR

930 CALL#1048        ' TRANSFER

940 NEXTS

950 NEXTT

960 END

/* Read sector for JASMIN + FT-DOS */

1000 POKE #48C,DR     ' DRIVE

1010 POKE #48D,T      ' TRACK

1020 POKE #48E,S      ' SECTOR

1030 DOKE #48F,#2000  ' BUFFER ADDRESS

1040 !RS              ' READ SECTOR

1050 RETURN

=== ORIC side of DISKLINK for other operating systems ==============

As should be clear by now, all there is to do is adapt the SETUP

section (first 5 lines) and provide a read sector routine.

If you want to read several types of disks with the same OS (e.g.

a SEDORIC disk with ORIC-DOS), you could modify the SETUP section

to interactively input the needed parameters :

1 DR=0    ' DRIVE

2 T0=0    ' TRACK  MIN

3 T1=39   ' TRACK  MAX

4 S0=1    ' SECTOR MIN

5 INPUT "SECTOR MAX ? "; S1

Therefore you could have S1=16 for an ORIC-DOS diskette and S1=17

for a SEDORIC diskette. As said before, the PC side automatically

adapts.

====================================================================

HAPPY TRANSFERRING !

====================================================================