This is OricTerm v1.0, a terminal emulator of something which has never

existed. It should run on every PC platform, from the earliest 8088 PC to

the most powerful. It is able to use about any serial communication port

of your PC, and about any graphics card since it only uses a CGA compatible

text mode.

Type oricterm -help to have a summary of the options, these options mainly

address the communication port setting, a switch for french characters (as

they are defined under Sedoric), a switch for dealing with the snow on

a CGA card, and a switch to remove your bad clock interrupt handler if you

experience character loss.

But what is an Oric terminal ? Simple, it's a terminal you hook to your oric !

The main benefits you get from its use are: a 80 column display, and ability

to drive two screens with your oric (great for remote debugging, or displaying

text and graphics at the same time, etc). You need a null-modem cable to

connect the terminal (PC) to the oric, or a straight cable plus a modem plus

a phone number where an oric is waiting (not very common, but...). In any

case, you need a serial port on the server (oric), plus a server software

package on it (not very common either...). Of course, IQ164/Stratos/Telestrat

have a serial port. Atmos'es need a serial extension card.

To test this software, if you have a Telestrat, you just need to issue a

"OPCH 0,16" command to redirect the output of the oric to the terminal (you

may close the normal screen output with "CLCH 0,8"), and a similar "OPCH 0,3"

command to allow the input flowing from the terminal (and "CLCH 0,0" if you

want to let the cat play with the oric keyboard). If you have an Atmos or an

Oric-1, you can drive the terminal with your own software or patch some

"deficiencies" in the rom (i.e. burn an eprom). I have given the bugfixes in

the CEO-mag, with simple routines driving the serial port.

State of the Oric terminal emulation (how can I emulate a chimera ?-)

------------------------------------

- conversion in both direction of some "oric ascii" characters: ,c,

and ,,,,, in "accent set" mode (if the previous characters in this

documentation look strange, use the original IBM-PC extended ascii set

to display)

- Oric and Telestrat control characters implemented, except double height.

- scrolling of screen when lines 25 to 27 are accessed.

- Some attributes implemented: background and foreground colors, flashing.

Due to implementation choices (making it run on a simple PC with CGA. Why

should a program requires a Pentium 200 when it can run on a simple PC ?),

the colors are not always the best ones (with CGA, you have the ability to

display highlighted background *or* flashing, but not both, so I have kept

flashing and yellow background is), and some attributes cannot be implemented

(double height for example).

Also, the scheme implemented for colors and flashing attributes is not perfect,

such attributes currently cause the rest of the line to be updated, without

regards of other existing attributes.

NEW in v1.1

-----------

- local handling of Ctrl-A, Backspace converted to Delete. Now you can

edit Basic programs as usual on the oric.

- local handling of Function keys: edit file userkeys.def to define your

favorite function keys. A very primitive handling of this file is done,

so be sure to only insert the character sequences you want, one line

for each function key (use the paragraph symbol to introduce Carriage Return

in your sequences). Note that the bundled file in this package initializes

every function key with its litteral string equivalent, so that you can

easily spot the line you want to modify. Be aware that your keyboard driver

or some resident programs may catch some of the function keys.

- hardware scrolling: 6845 register programming is now used. So the display

is much faster now, 8088 PCs win a second life ;-)

TO DO

-----

- perfect handling of the colors and flashing attributes

- suppress this annoying ^C echo

- option to select a different handling of the attributes, similar to ANSI.

- approximation of the alternate charset (with special 160x100 mode ?)

POLICY

------

- free software by F.Frances. Use at your own risk, I cannot be held

responsible if a nasty bug causes a tree in your garden to fall onto your

house.

- source is included, so feel free to modify this program. Note that the

assembly calling conventions used means you need the Watcom C compiler.